EXPERIMENT 6 — DIVISION OF 16-BIT NUMBERS USING 8086 MICROPROCESSOR

1. Aim

To write and execute an assembly language program to divide two 16-bit unsigned numbers using the 8086 microprocessor.

2. Apparatus Required

- 8086 microprocessor kit — 1

- Power card — 1

- Keyboard — 1

3. Algorithm

- Initialize the DX register with 0000H.

- Load the dividend from memory location 1200H into AX.

- Load the divisor from memory location 1202H into BX.

- Divide the 32-bit value DX:AX by BX.

- Store the quotient from AX at memory location 1204H.

- Store the remainder from DX at memory location 1206H.

- Stop the program.

4. Assembly Language Program — Corrected Code

MOV DX, 0000H

MOV AX, [1200H]

MOV BX, [1202H]

DIV BX

MOV [1204H], AX

MOV [1206H], DX

HLT

Important correction: The source document contains DIV CX, but this is inconsistent with the divisor being loaded into BX. The correct instruction is DIV BX.

5. Code Explanation

| Instruction | Explanation |
| --- | --- |
| MOV DX, 0000H | Clears DX before division. |
| MOV AX, [1200H] | Loads the dividend into AX. |
| MOV BX, [1202H] | Loads the divisor into BX. |
| DIV BX | Divides DX:AX by BX. |
| MOV [1204H], AX | Stores the quotient. |
| MOV [1206H], DX | Stores the remainder. |
| HLT | Stops program execution. |

6. Input and Output

Input memory map

| Memory address | Data |
| --- | --- |
| 1200H | EF12H |
| 1202H | ABCDH |

Expected output memory map

| Memory address | Data | Description |
| --- | --- | --- |
| 1204H | 0001H | Quotient |
| 1206H | 4345H | Remainder |

7. Calculation

Dividend = EF12H

Divisor = ABCDH

\[ \text{Quotient}=\left\lfloor \frac{\text{EF12H}}{\text{ABCDH}} \right\rfloor=0001H \]

\[ \text{Remainder}=\text{EF12H}-\text{ABCDH} =\text{4345H} \]

Therefore:

- Quotient: 0001H

- Remainder: 4345H

8. Procedure

- Switch on the 8086 microprocessor kit.

- Enter the program into the kit or simulator.

- Enter the dividend EF12H at memory location 1200H.

- Enter the divisor ABCDH at memory location 1202H.

- Execute the program.

- Examine memory location 1204H for the quotient.

- Examine memory location 1206H for the remainder.

- Verify the output against the expected values.

9. Result

Thus, the 8086 assembly language program for division of two 16-bit unsigned numbers was written and executed. The quotient 0001H and remainder 4345H were obtained and stored in the specified memory locations.