EXPERIMENT 5 — 16-BIT MULTIPLICATION USING 8086 MICROPROCESSOR

1. Aim

To write and execute an 8086 assembly language program to multiply two 16-bit numbers and store the 32-bit result in memory.

2. Software Required

- 8086 Microprocessor simulator

- Assembly language programming environment

3. Assembly Language Program

MOV AX, [1200h]

MOV BX, [1202h]

MUL BX

MOV [1204h], AX

MOV [1206h], DX

HLT

4. Input

| Memory address | Input value |
| --- | --- |
| 1200H | ABCDH |
| 1202H | EF12H |

- Open the 8086 microprocessor simulator.

- Create a new project or program.

- Enter the assembly language code.

- Store the first 16-bit number at memory address 1200H.

- Store the second 16-bit number at memory address 1202H.

- Assemble the program and check for errors.

- Run the program step by step or execute it.

- Examine memory locations 1204H and 1206H to obtain the result.

6. Code Explanation

- MOV AX, [1200h] — Loads the first 16-bit number into register AX.

- MOV BX, [1202h] — Loads the second 16-bit number into register BX.

- MUL BX — Multiplies AX by BX. The 32-bit result is stored in DX:AX.

- MOV [1204h], AX — Stores the lower 16 bits of the result.

- MOV [1206h], DX — Stores the higher 16 bits of the result.

- HLT — Stops program execution.

7. Expected Output

| Memory address | Output value | Description |
| --- | --- | --- |
| 1204H | 776AH | Lower 16 bits |
| 1206H | A070H | Higher 16 bits |

The complete 32-bit result is A070776AH.

8. Result

Thus, the 16-bit multiplication program was executed using the 8086 microprocessor, and the 32-bit result was stored in the specified memory locations.

Note: The program and memory addresses follow the Experiment 5 details retrieved from your lab-record context. The expected output should be checked against the exact input values and program listing in your record.