Experiment 7: 32-Bit Addition Using 8086 Microprocessor

1. Aim

To write and execute an assembly language program to add two 32-bit numbers using the 8086 microprocessor and store the 32-bit result in memory.

2. Objective

- To perform addition of two 32-bit numbers.

- To understand the use of ADD and ADC instructions.

- To store the lower and higher words of the result in memory.

- To verify the addition result using an 8086 emulator.

3. Apparatus / Software Required

- 8086 microprocessor emulator (EMU8086)

- Computer or laptop

- 8086 assembly language editor and emulator

4. Algorithm

- Initialize the data segment.

- Load the lower word of the first number into the AX register.

- Add the lower word of the second number using the ADD instruction.

- Store the lower-word result in memory.

- Load the higher word of the first number into AX.

- Add the higher word of the second number and the carry, if any, using the ADC instruction.

- Store the higher-word result in memory.

- Stop program execution.

5. Assembly Language Program (EMU8086)

.MODEL SMALL

.STACK 100H

.DATA

NUM1_LOW  DW 5678H

NUM1_HIGH DW 1234H

NUM2_LOW  DW 5678H

NUM2_HIGH DW 1234H

RES_LOW   DW ?

RES_HIGH  DW ?

.CODE

MAIN PROC

MOV AX, @DATA

MOV DS, AX

MOV AX, NUM1_LOW

ADD AX, NUM2_LOW

MOV RES_LOW, AX

MOV AX, NUM1_HIGH

ADC AX, NUM2_HIGH

MOV RES_HIGH, AX

MOV AH, 4CH

INT 21H

MAIN ENDP

END MAIN

6. Input and Output

Input:

| Variable | Value |
| --- | --- |
| NUM1_LOW | 5678H |
| NUM1_HIGH | 1234H |
| NUM2_LOW | 5678H |
| NUM2_HIGH | 1234H |

Output:

| Variable | Value | Description |
| --- | --- | --- |
| RES_LOW | ACF0H | Lower 16-bit result |
| RES_HIGH | 2468H | Higher 16-bit result |

Calculation:

\[ 12345678_H+12345678_H=2468ACF0_H \]

Therefore, the final 32-bit result is 2468ACF0H.

7. Procedure

- Open the EMU8086 application.

- Create a new assembly language program.

- Enter the corrected code in the editor.

- Click Emulate to assemble and load the program.

- Run the program.

- Inspect the variables RES_LOW and RES_HIGH in the emulator.

- Verify that the lower word is ACF0H and the higher word is 2468H.

8. Result

Thus, the assembly language program for adding two 32-bit numbers using the 8086 microprocessor was written and executed successfully. The result, 2468ACF0H, was verified using the EMU8086 emulator.